# ESP32 Relay Control with Web Button

This project demonstrates how to control a relay module (and hence a water pump) using an ESP32 through a simple web interface. The relay is controlled using an ON/OFF button from a laptop or smartphone connected to the same Wi-Fi network.

# Components Required

- ESP32 Development Board

- Relay Module (5V)

- Water Pump (connected through relay)

- MB102 Breadboard Power Supply (for pump if required)

- Jumper Wires

- Laptop/Smartphone (for web control)

# Wiring Connections

ESP32 → Relay Module:

- GPIO4 (D4) → IN (Relay Signal Pin)

- 5V (ESP32) → VCC (Relay)

- GND (ESP32) → GND (Relay)

Relay Module → Water Pump:

- COM → Pump positive terminal

- NO → Power Supply + (12V or as per pump rating)

- Pump negative terminal → Power Supply - (GND)

# Arduino Code

#include <WiFi.h>
#include <WebServer.h>

const char* ssid = "YOUR_WIFI_SSID";
const char* password = "YOUR_WIFI_PASSWORD";

#define RELAY_PIN 4

WebServer server(80);

void handleRoot() {
  server.send(200, "text/html",
              "<h1>ESP32 Relay Control</h1>"
              "<button onclick=\"location.href='/on'\">ON</button>"
              "<button onclick=\"location.href='/off'\">OFF</button>");
}

void handleOn() {
  digitalWrite(RELAY_PIN, LOW);
  server.send(200, "text/html", "<h1>Relay ON</h1><a href='/'>Back</a>");
}

void handleOff() {
  digitalWrite(RELAY_PIN, HIGH);
  server.send(200, "text/html", "<h1>Relay OFF</h1><a href='/'>Back</a>");
}

void setup() {
  pinMode(RELAY_PIN, OUTPUT);
  digitalWrite(RELAY_PIN, HIGH);
  Serial.begin(115200);

  WiFi.begin(ssid, password);
  while (WiFi.status() != WL_CONNECTED) {
    delay(1000);
    Serial.println("Connecting to WiFi...");
  }
  Serial.println(WiFi.localIP());

  server.on("/", handleRoot);
  server.on("/on", handleOn);
  server.on("/off", handleOff);
  server.begin();
}

void loop() {
  server.handleClient();
}

#include <WiFi.h>

#include <WebServer.h>

// Relay Pins

#define RELAY_PUMP 4   // Pump Relay (D4 → GPIO4)

#define RELAY_FAN  5   // Fan Relay (D5 → GPIO5)

// WiFi credentials

const char* ssid = "BSNL FIBER 5G";

const char* password = "944378850000";

// Web server

WebServer server(80);

// Relay states (active LOW relay)

bool pumpState = HIGH;

bool fanState  = HIGH;

// HTML Page

String htmlPage() {

String html = "<!DOCTYPE html><html><head>";

html += "<meta name='viewport' content='width=device-width, initial-scale=1.0'>";

html += "<title>ESP32 Device Control</title>";

html += "<style>";

html += "body{font-family: Arial; text-align:center; margin-top:30px;}";

html += ".btn{padding:15px; font-size:18px; margin:10px; border:none; border-radius:10px; cursor:pointer;}";

html += ".on{background:green; color:white;}";

html += ".off{background:red; color:white;}";

html += "</style></head><body>";

html += "<h1>ESP32 Water Pump & Fan Control</h1>";

// Pump status

html += "<h2>Pump is currently: <b>" + String(pumpState == LOW ? "ON" : "OFF") + "</b></h2>";

if (pumpState == LOW) {

html += "<a href='/pump_off'><button class='btn off'>Turn Pump OFF</button></a>";

} else {

html += "<a href='/pump_on'><button class='btn on'>Turn Pump ON</button></a>";

}

// Fan status

html += "<h2>Fan is currently: <b>" + String(fanState == LOW ? "ON" : "OFF") + "</b></h2>";

if (fanState == LOW) {

html += "<a href='/fan_off'><button class='btn off'>Turn Fan OFF</button></a>";

} else {

html += "<a href='/fan_on'><button class='btn on'>Turn Fan ON</button></a>";

}

html += "</body></html>";

return html;

}

// Handlers

void handleRoot() {

server.send(200, "text/html", htmlPage());

}

void handlePumpOn() {

pumpState = LOW;

digitalWrite(RELAY_PUMP, pumpState);

server.send(200, "text/html", htmlPage());

}

void handlePumpOff() {

pumpState = HIGH;

digitalWrite(RELAY_PUMP, pumpState);

server.send(200, "text/html", htmlPage());

}

void handleFanOn() {

fanState = LOW;

digitalWrite(RELAY_FAN, fanState);

server.send(200, "text/html", htmlPage());

}

void handleFanOff() {

fanState = HIGH;

digitalWrite(RELAY_FAN, fanState);

server.send(200, "text/html", htmlPage());

}

void setup() {

Serial.begin(115200);

pinMode(RELAY_PUMP, OUTPUT);

pinMode(RELAY_FAN, OUTPUT);

digitalWrite(RELAY_PUMP, HIGH);  // OFF initially

digitalWrite(RELAY_FAN, HIGH);   // OFF initially

// WiFi setup

WiFi.begin(ssid, password);

Serial.println("Connecting to WiFi...");

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.print(".");

}

Serial.println("\nConnected!");

Serial.print("ESP32 IP Address: ");

Serial.println(WiFi.localIP());

// Web routes

server.on("/", handleRoot);

server.on("/pump_on", handlePumpOn);

server.on("/pump_off", handlePumpOff);

server.on("/fan_on", handleFanOn);

server.on("/fan_off", handleFanOff);

server.begin();

}

void loop() {

server.handleClient();

}

| Device | ESP32 Pin |
| --- | --- |
| Pump Relay | GPIO25 |
| Fan Relay | GPIO26 |
| TFT CS | GPIO15 |
| TFT DC | GPIO2 |
| TFT RST | GPIO4 |
| TFT SCLK | GPIO18 |
| TFT MOSI | GPIO23 |
| TFT VCC | 3.3V/5V |
| TFT GND | GND |
| TFT BLK | 3.3V |

Output

Positive   blue,violet

Negative  yellow,green